Radio transmitter control quiz

Student Name ______________________ Date __________ Class Period _________

1. A transmitter is using a crystal having a resonant frequency of 75.49 megahertz. A receiver has a crystal with a resonant frequency of 79.41 megahertz. The transmitter frequency is close enough to activate the receiver.

A. True

B. False

2. Resonance phenomena only occur in electronic circuits.

A. True

B. False

3. The RF signal from a Vex transmitter is transmitted in:

A. only to the left of the remote control operator

B. only to the right of the remote control operator

C. only in an axial direction along the antenna length

D. only in a radial direction along the antenna length

E. none of the above

4. A fresh battery is critical to the remote system when conducting experimental investigations.

A. True

B. False

5. The size and shape of the Vex transmitter crystal and the Vex receiver crystal are:

A. Equal

B. The receiver crystal is larger than the transmitter crystal

C. The receiver crystal is smaller than the transmitter crystal

D. None of the above

6. When using the remote control to run a Vex robot, the following items are important:

A. antenna position only

B. distance to the receiver only

C. battery power in the transmitter only

D. all of the above are important

E. none of the above are important

7. The time period for a 75.93 MHz Crystal is: (you must use a calculator to answer)

A. 7.593 microseconds

B. 75.93 nanoseconds

T=1f	C. 13.17 nanoseconds

D. 1.317 nanoseconds

E. 131.7 microseconds

8. The frequency for a crystal having a time period of 2.5 microseconds is:

A. 250 megahertz

f=1T	B. 25 kilohertz

C. 4 megahertz

D. 400 kilohertz

E. 4 gigahertz

9. The difference between a 75.41 megahertz crystal and a 75.49 megahertz crystal is:

A. so small it is meaningless

B. .08 megahertz

C. 80 kilohertz

D. A and B

E. B and C

10. The time period difference between a 75. 89 megahertz crystal and a 75.49 megahertz crystal is:

A. so small it is meaningless

B. 1.25 microseconds

C. 12.5 microseconds

D. 0.4 nanoseconds

E. 12. 5 megahertz